# COMPUTER NETWORKS (CSA0716)

## Assessment Tool 1 – Short Answer Test Solutions

Name: Naga Jaswanth B 
Register Number: 192424300

QUESTION 1

Given:

- File size = 150 MB = 150,000,000 bytes

- Segment size = 1500 bytes

- Data Link Layer overhead = 40 bytes per frame

- Bandwidth = 50 Mbps

Calculation

Step 1: Calculate the number of segments

Number of segments = File size ÷ Segment size

= 150,000,000 ÷ 1500

= 100,000 segments

Since one segment is placed into one frame,

Number of frames = 100,000 frames

Step 2: Calculate the total Data Link Layer overhead

Total overhead = Number of frames × Overhead per frame

= 100,000 × 40

= 4,000,000 bytes

= 4 MB

Step 3: Calculate the total data transmitted

Total data transmitted

= Original file size + Overhead

= 150 MB + 4 MB

= 154 MB

Step 4: Calculate the transmission time

Convert MB into Megabits

154 × 8

= 1232 Megabits

Transmission time

= Total bits ÷ Bandwidth

= 1232 ÷ 50

= 24.64 seconds

Answer

- Number of segments = 100,000

- Number of frames = 100,000

- Total overhead = 4 MB

- Transmission time = 24.64 seconds

Explanation

The Transport Layer divides the file into smaller segments and ensures reliable delivery using sequence numbers, acknowledgements, retransmissions, and flow control. The Data Link Layer encapsulates each segment into frames by adding headers and trailers, performs error detection using CRC, and ensures reliable communication between directly connected devices. Together, these layers provide accurate, efficient, and reliable data transmission.

QUESTION 2

Given:

- Sliding window size = 6 frames

- Total frames = 48

- Frame size = 1024 bytes

Calculation

Step 1: Calculate the number of transmission windows

Number of windows

= Total frames ÷ Window size

= 48 ÷ 6

= 8 windows

Step 2: Calculate retransmissions

One frame is lost in every window.

Total retransmissions

= 8 × 1

= 8 frames

Answer

- Number of transmission windows = 8

- Total retransmissions = 8 frames

Explanation

Flow control regulates the rate of data transmission between the sender and receiver. It prevents receiver buffer overflow, minimizes packet loss, avoids unnecessary retransmissions, and improves communication efficiency. The sliding window protocol allows multiple frames to be transmitted before receiving acknowledgements, thereby increasing network throughput.

QUESTION 3

Given:

- IP packet size = 12,000 bytes

- MTU = 1500 bytes

- IP header = 20 bytes

Calculation

Maximum payload in one fragment

= MTU − Header

= 1500 − 20

= 1480 bytes

Payload to be fragmented

= 12,000 − 20

= 11,980 bytes

Number of fragments

= 11,980 ÷ 1480

= 8.09

Therefore,

= 9 fragments

Header overhead

= Number of fragments × Header size

= 9 × 20

= 180 bytes

Answer

- Number of fragments = 9

- Total header overhead = 180 bytes

Explanation

The Network Layer performs fragmentation whenever a packet exceeds the MTU of the network. Each fragment contains its own IP header with identification, fragment offset, and control information. At the destination, these fragments are reassembled correctly to reconstruct the original packet before it is delivered to the upper layers.

QUESTION 4

Given:

- Sliding window size = 6 frames

- Total frames = 48

Calculation

Step 1: Number of windows

48 ÷ 6

= 8 windows

Step 2: Retransmissions

One frame is lost in every window.

Total retransmissions

= 8 × 1

= 8 frames

Answer

- Number of windows = 8

- Total retransmissions = 8 frames

Explanation

Flow control synchronizes the transmission speed between the sender and receiver. It prevents congestion, reduces frame loss, improves bandwidth utilization, and ensures efficient communication using the sliding window protocol.

QUESTION 5

Given:

- Video size = 600 MB

- Synchronization checkpoint every 60 MB

- Failure occurs after 390 MB

Calculation

Step 1: Calculate completed checkpoints

390 ÷ 60

= 6.5

Completed checkpoints

= 6

Step 2: Last successful checkpoint

6 × 60

= 360 MB

Step 3: Data to be retransmitted

390 − 360

= 30 MB

Answer

- Number of checkpoints created = 6

- Data retransmitted after recovery = 30 MB

Explanation

The Session Layer establishes, manages, and terminates communication sessions. Synchronization checkpoints divide long transmissions into recovery points. When a failure occurs, transmission resumes from the last checkpoint instead of restarting from the beginning. This reduces retransmission time, saves bandwidth, and improves communication reliability.

QUESTION 6

Given:

- Original file size = 900 MB

- Compression = 40%

- Encryption overhead = 5% of the compressed file size

Calculation

Step 1: Calculate the compressed file size

Compressed file size

= Original file size × (100 − Compression) ÷ 100

= 900 × 60 ÷ 100

= 540 MB

Step 2: Calculate the encryption overhead

Encryption overhead

= 540 × 5 ÷ 100

= 27 MB

Step 3: Calculate the encrypted file size

Encrypted file size

= Compressed file size + Encryption overhead

= 540 + 27

= 567 MB

Step 4: Calculate the total percentage reduction

Percentage reduction

= ((Original file size − Encrypted file size) ÷ Original file size) × 100

= ((900 − 567) ÷ 900) × 100

= (333 ÷ 900) × 100

= 37%

Answer

- Compressed file size = 540 MB

- Encrypted file size = 567 MB

- Total percentage reduction = 37%

Explanation

The Presentation Layer is responsible for data compression, encryption, and format conversion. Compression reduces the amount of data transmitted, improving bandwidth utilization. Encryption protects data from unauthorized access during transmission. Thus, the Presentation Layer enhances both communication efficiency and security.

QUESTION 7

Given:

- Data transferred = 3 GB

- Effective throughput = 120 Mbps

- Request size = 3 MB

Calculation

Step 1: Convert GB into MB

3 GB = 3000 MB

Step 2: Calculate the number of requests

Number of requests

= Total data ÷ Request size

= 3000 ÷ 3

= 1000 requests

Step 3: Calculate the transmission time

Convert data into Megabits

3000 MB × 8

= 24,000 Megabits

Transmission time

= Total Megabits ÷ Throughput

= 24,000 ÷ 120

= 200 seconds

Answer

- Number of requests = 1000

- Transmission time = 200 seconds

Explanation

The Application Layer provides network services directly to end users. FTP enables reliable file transfer by managing file requests, user authentication, and communication between the client and server. It ensures accurate, secure, and efficient file transfer over the network.

QUESTION 8

Given:

- Application Layer delay = 4 ms

- Transport Layer delay = 3 ms

- Network Layer delay = 5 ms

- Data Link Layer delay = 2 ms

- Physical Layer delay = 1 ms

- Propagation delay = 18 ms

- Transmission delay = 12 ms

Calculation

Step 1: Calculate the total processing delay

Processing delay

= 4 + 3 + 5 + 2 + 1

= 15 ms

Step 2: Calculate the total end-to-end delay

Total delay

= Processing delay + Propagation delay + Transmission delay

= 15 + 18 + 12

= 45 ms

Answer

- Processing delay = 15 ms

- Total end-to-end delay = 45 ms

Explanation

Each OSI layer contributes to successful communication. The Application Layer provides user services, the Transport Layer ensures reliable delivery, the Network Layer performs routing, the Data Link Layer manages framing and error detection, and the Physical Layer transmits bits through the communication medium. Together, these layers determine the total communication delay.

QUESTION 9

Given:

- Useful data = 80 MB = 80,000,000 bytes

- Frame size = 1600 bytes

- Protocol overhead = 80 bytes per frame

Calculation

Step 1: Calculate the useful data per frame

Useful data per frame

= Frame size − Overhead

= 1600 − 80

= 1520 bytes

Step 2: Calculate the number of frames

Number of frames

= Total useful data ÷ Useful data per frame

= 80,000,000 ÷ 1520

= 52,631.58

≈ 52,632 frames

Step 3: Calculate the total protocol overhead

Total overhead

= Number of frames × Overhead per frame

= 52,632 × 80

= 4,210,560 bytes

Step 4: Calculate the transmission efficiency

Transmission efficiency

= (Useful data per frame ÷ Frame size) × 100

= (1520 ÷ 1600) × 100

= 0.95 × 100

= 95%

Answer

- Number of frames = 52,632

- Total protocol overhead = 4,210,560 bytes

- Transmission efficiency = 95%

Explanation

Protocol overhead consists of headers and trailers added by different OSI layers. Although overhead reduces the effective data rate slightly, it is necessary for addressing, routing, error detection, and reliable communication. Efficient protocol design minimizes overhead while maintaining network performance and reliability.

QUESTION 10

Given:

- Original medical record size = 240 MB

- Compression = 30%

- Segment size = 1400 bytes

- Data Link Layer overhead = 60 bytes per frame

Calculation

Step 1: Calculate the compressed file size

Compressed file size

= Original size × (100 − Compression) ÷ 100

= 240 × 70 ÷ 100

= 168 MB

Step 2: Convert compressed file into bytes

168 MB

= 168,000,000 bytes

Step 3: Calculate the number of segments

Number of segments

= 168,000,000 ÷ 1400

= 120,000 segments

Step 4: Calculate the total protocol overhead

Protocol overhead

= Number of segments × Overhead per frame

= 120,000 × 60

= 7,200,000 bytes

= 7.2 MB

Step 5: Calculate the final amount of data transmitted

Final transmitted data

= Compressed file size + Protocol overhead

= 168 + 7.2

= 175.2 MB

Answer

- Compressed file size = 168 MB

- Number of segments = 120,000

- Total protocol overhead = 7.2 MB

- Final data transmitted = 175.2 MB

Explanation

The Presentation Layer compresses the medical record to reduce its size and may encrypt it to ensure confidentiality. The Transport Layer divides the compressed data into segments for reliable transmission. The Data Link Layer encapsulates each segment into frames by adding headers and trailers for error detection. Finally, the Physical Layer transmits the data through the communication medium. Together, these layers provide secure, reliable, and efficient communication in an IoT-enabled healthcare network.